Häme University of Applied Sciences

Spring 2026

Smart Commercial Refrigerator

Thinger.io Cloud Dashboard

Setup Guide

Rajan Basnet  |  Narges Khalafi  |  Fardush Bappy  |  Maryam Davari

# 1. Create Your Thinger.io Account

- Go to thinger.io in your browser.

- Click Sign Up. Fill in your email, username, and password.

- Check your email for a confirmation link and click it.

- Log in. You land on the main console.

# 2. Create Two Devices

You need one device for the Dasduino (main controller) and one for the ESP32-CAM. Each device gets its own token that you paste into the firmware config.h files.

## Device 1 — Main Controller

- In the left sidebar, click Devices.

- Click the blue + button to add a new device.

- Fill in the form:

| Field | Value |
| --- | --- |
| Device Id | smart_fridge_main |
| Device credentials | Auto-generated — copy this token |
| Device description | Dasduino WROVER-E main controller |

- Click Add Device.

- Copy the Device Credentials token. Paste it into THINGER_TOKEN in main_esp32/include/config.h.

## Device 2 — Camera Controller

- Click the + button again.

- Fill in the form:

| Field | Value |
| --- | --- |
| Device Id | smart_fridge_cam |
| Device credentials | Auto-generated — copy this token |
| Device description | AI Thinker ESP32-CAM |

- Click Add Device.

- Copy the token. Paste it into THINGER_TOKEN in esp32cam/include/config.h.

# 3. Create the Dashboard

- In the left sidebar, click Dashboards.

- Click the blue + button.

- Set Dashboard Id to smart_fridge_dashboard.

- Set a display name: Smart Fridge Monitor.

- Click Add Dashboard.

- Click the dashboard name to open it.

- Click the pencil icon (Edit mode) to start adding widgets.

# 4. Add Dashboard Widgets

Add each widget below one by one. For each widget, click the + Add Widget button, select the type, and fill in the settings shown.

## Widget 1 — Temperature Gauge

| Setting | Value |
| --- | --- |
| Widget type | Gauge |
| Widget title | Temperature |
| Device | smart_fridge_main |
| Resource | temperature |
| Min value | 0 |
| Max value | 20 |
| Units | C |

## Widget 2 — Humidity Gauge

| Setting | Value |
| --- | --- |
| Widget type | Gauge |
| Widget title | Humidity |
| Device | smart_fridge_main |
| Resource | humidity |
| Min value | 0 |
| Max value | 100 |
| Units | % |

## Widget 3 — Weight Chart

| Setting | Value |
| --- | --- |
| Widget type | Time Series Chart |
| Widget title | Shelf Weight |
| Device | smart_fridge_main |
| Resource | weight_kg |
| Units | kg |

## Widget 4 — Door Status

| Setting | Value |
| --- | --- |
| Widget type | LED Indicator |
| Widget title | Door |
| Device | smart_fridge_main |
| Resource | door_open |
| On label | OPEN |
| Off label | CLOSED |
| On colour | Red |
| Off colour | Green |

## Widget 5 — Door Alert

| Setting | Value |
| --- | --- |
| Widget type | LED Indicator |
| Widget title | Door Alert |
| Device | smart_fridge_main |
| Resource | door_alert |
| On label | ALERT — Door open too long |
| Off label | OK |
| On colour | Red |
| Off colour | Green |

## Widget 6 — Temperature Alert

| Setting | Value |
| --- | --- |
| Widget type | LED Indicator |
| Widget title | Temp Alert |
| Device | smart_fridge_main |
| Resource | temp_alert |
| On label | TEMP TOO HIGH |
| Off label | OK |
| On colour | Red |
| Off colour | Green |

## Widget 7 — OCR Result

| Setting | Value |
| --- | --- |
| Widget type | Text / Value |
| Widget title | Last Scan Result |
| Device | smart_fridge_cam |
| Resource | ocr_result |

## Widget 8 — Capture Button

| Setting | Value |
| --- | --- |
| Widget type | Button |
| Widget title | Take Photo |
| Device | smart_fridge_cam |
| Resource | capture |
| Button label | Capture |

Click Save on the dashboard after adding all widgets.

# 5. Create Data Buckets

Data buckets store historical sensor readings so you can view trends over time.

- In the left sidebar, click Data Buckets.

- Click the + button. Create three buckets with these settings:

| Bucket Id | Device | Resource | Sampling |
| --- | --- | --- | --- |
| fridge_temperature | smart_fridge_main | temperature | Every 5 minutes |
| fridge_humidity | smart_fridge_main | humidity | Every 5 minutes |
| fridge_weight | smart_fridge_main | weight_kg | Every 5 minutes |

# 6. Create an Alert (Email Notification)

This sends an email to the store manager when the door has been open too long.

- In the left sidebar, click Endpoints.

- Click the + button.

- Set Endpoint Id to door_alert_email.

- Set Endpoint Type to Email.

- Fill in the email address that should receive alerts.

- Set Subject to: Smart Fridge — Door Alert.

- Set Body to: The refrigerator door has been open for more than 30 seconds. Please check immediately.

- Click Add Endpoint.

- Now click Properties in the left sidebar.

- Find the Rules or Server Events section.

- Create a rule: if smart_fridge_main.door_alert equals true, call door_alert_email endpoint.

You will now receive an email whenever the door stays open longer than 30 seconds.

# 7. Verify Everything Works

- Power on the Dasduino WROVER-E with the main firmware flashed.

- Open the Thinger.io dashboard. Within 5 seconds, temperature and humidity widgets should show live values.

- Press the tactile switch by hand. The Door LED on the dashboard should turn red.

- Release the switch. The Door LED should turn green.

- Hold the switch open for 30 seconds. The Door Alert LED should turn red and you should receive the email.

- Press the Capture button. Within 10 seconds, the OCR result widget should show text.