Häme University of Applied Sciences

Spring 2026

Smart Commercial Refrigerator

System Testing and Debugging

Test Guide

Rajan Basnet  |  Narges Khalafi  |  Fardush Bappy  |  Maryam Davari

# 1. How to Use This Guide

Work through each section in order. Each test has a pass condition. If a test fails, follow the debug steps before moving to the next section. Do not proceed to the next component until the current one passes.

Open Serial Monitor in VS Code PlatformIO at 115200 baud for all tests on the Dasduino. Look for the prefix codes in each log line: [OK], [WARN], [ALERT], [DOOR], [CAM].

# 2. Pre-Test Checklist

Confirm each item before starting any test:

| Item | Check |
| --- | --- |
| VS Code and PlatformIO installed |  |
| main_esp32 project builds without errors |  |
| esp32cam project builds without errors |  |
| config.h files filled in (Wi-Fi, Thinger.io tokens) |  |
| TFT_eSPI_User_Setup.h copied to library folder |  |
| DHT22 wired to GPIO 4 with 10kOhm pull-up to 3.3V |  |
| Tactile switch: one leg to GPIO 5, other leg to GND |  |
| HX711 DT on GPIO 25, SCK on GPIO 26 (NOT 16/17) |  |
| Load cell wires: Red E+, Black E-, Green A+, White A- |  |
| TFT wired: CS=14, DC=2, RST=15, MOSI=23, SCLK=18 |  |
| ESP32-CAM ribbon cable seated (metal contacts down) |  |

# 3. Test 1 — DHT22 Temperature and Humidity

## What to do

- Flash main_esp32 to the Dasduino.

- Open Serial Monitor at 115200.

- Watch for DHT22 log lines.

## Pass condition

- Serial Monitor shows [OK] DHT22 initialised on startup.

- Temperature reads between 15C and 30C at room temperature.

- Humidity reads between 20% and 80% at room temperature.

- No [WARN] DHT22 read failed messages appear.

## If it fails

| Symptom | Fix |
| --- | --- |
| [WARN] DHT22 read failed on every loop | Check 10kOhm pull-up between VCC and DATA. Confirm VCC is 3.3V not 5V. |
| Temperature reads 0.0 constantly | DATA wire is disconnected or GPIO 4 is wrong. |
| Temperature reads -40.0 | Pull-up resistor is missing. GPIO pin is floating. |
| Random values jumping wildly | Cable too long (over 20cm) or interference. Shorten the wire. |

# 4. Test 2 — TFT Display

## What to do

- With main_esp32 running, look at the TFT screen.

## Pass condition

- Screen shows temperature, humidity, weight, and door state.

- Values update every 2 seconds.

- No flickering, no black screen, no garbled text.

## If it fails

| Symptom | Fix |
| --- | --- |
| Screen is completely black | Check all 5 SPI wires (CS, DC, RST, MOSI, SCLK). Confirm VCC is 3.3V. |
| Screen shows random coloured blocks | Wrong driver. Confirm ST7789_DRIVER in User_Setup.h. |
| Text shows but colours are wrong | Change TFT_RGB_ORDER from TFT_RGB to TFT_BGR in User_Setup.h. |
| Build error mentioning TFT_eSPI | User_Setup.h not in the right folder. Must be in .platformio/lib/TFT_eSPI/. |
| Screen flickers every 2 seconds | fillScreen() clears everything. This is normal. Reduce update frequency if needed. |

# 5. Test 3 — Door Switch

## What to do

- With main_esp32 running, press and release the tactile switch by hand.

## Pass condition

- Serial Monitor shows [DOOR] Opened when you release the button.

- Serial Monitor shows [DOOR] Closed when you press it.

- TFT shows OPEN when released, SHUT when pressed.

- Holding it released for 30 seconds triggers [ALERT] Door open too long and DOOR ALERT on TFT.

## If it fails

| Symptom | Fix |
| --- | --- |
| Door state never changes | Confirm one leg on GPIO 5, other on GND. Check INPUT_PULLUP in firmware. |
| Door alert fires instantly | Switch is wired backwards (legs swapped) or GPIO 5 is floating. |
| Door state flickers rapidly | Switch bounce. Increase DEBOUNCE_MS from 10 to 50 in config.h. |
| Dashboard does not reflect door state | Thinger.io not connected. Check Wi-Fi credentials and THINGER_TOKEN. |

# 6. Test 4 — Thinger.io Cloud Connection

## What to do

- Open the Thinger.io dashboard in your browser.

- Power on the Dasduino with main firmware.

## Pass condition

- Temperature and humidity gauges show live values within 5 seconds.

- Values update every 2 seconds.

- Door LED reflects physical switch state in real time.

## If it fails

| Symptom | Fix |
| --- | --- |
| Dashboard shows no data | Check WIFI_SSID, WIFI_PASSWORD, THINGER_USER, THINGER_TOKEN in config.h. |
| Device shows offline in Thinger.io | Firmware is not reaching Thinger.io servers. Try a different Wi-Fi network. |
| Data appears then stops | thing.handle() is being blocked. Remove any delay() calls from loop(). |
| Wrong values on dashboard | Resource names must match exactly: temperature, humidity, weight_kg, door_open. |

# 7. Test 5 — ESP32-CAM

## What to do

- Flash esp32cam firmware to the ESP32-CAM via the MB board.

- Press RST. Open Serial Monitor at 115200.

- Press the Capture button on the Thinger.io dashboard.

## Pass condition

- Serial Monitor shows [OK] Camera initialised on startup.

- Serial Monitor shows [OK] Camera controller ready.

- After pressing Capture, Serial Monitor shows [CAM] Capturing...

- Within 10 seconds, [OCR] Result appears in Serial Monitor.

- OCR result appears in the Thinger.io dashboard text widget.

## If it fails

| Symptom | Fix |
| --- | --- |
| [ERROR] Camera init failed | Ribbon cable not seated. Open locking tab, reinsert (metal contacts down), close tab. |
| Upload timeout in PlatformIO | Redo IO0+RST sequence. Hold IO0 a full second before pressing RST. |
| Camera ready but no capture response | Thinger.io capture resource not matching. Check THINGER_DEVICE matches smart_fridge_cam. |
| HTTP 403 from Google Vision | API key invalid or Cloud Vision API not enabled in Google Cloud Console. |
| HTTP 400 from Google Vision | Image too large or malformed. Lower jpeg_quality number (try 15) in firmware. |
| OCR result is empty or No text found | Label too far away, too dark, or blurry. Move camera to 10-15cm from label. Check flash fires. |
| OCR result is garbage characters | Image is overexposed. Reduce FLASH_DELAY_MS from 300 to 150 in esp32cam config.h. |

# 8. Test 6 — Load Cell (Run Last)

## What to do

- Flash calibration sketch. Get calibration factor. Update config.h. Re-flash main_esp32.

- Place a known weight on the shelf.

## Pass condition

- Weight reads within 5% of true value.

- Weight reads 0.00 kg when shelf is empty (after tare).

- Weight updates in real time on TFT and Thinger.io dashboard.

## If it fails

| Symptom | Fix |
| --- | --- |
| Weight always reads 0.00 | HX711 not initialised. Confirm DT on GPIO 25, SCK on GPIO 26. |
| Weight reads negative | Swap A+ and A- wires on HX711. |
| Weight drifts over time | Temperature compensation needed. Re-tare before each session. |
| Weight jumps around randomly | Electrical noise. Use shielded cable between load cell and HX711. Keep away from power lines. |
| Weight reads a fixed non-zero value | Tare not called. Press the tare_scale button on Thinger.io dashboard or reflash. |

# 9. 24-Hour Stability Test

Run this after all individual tests pass. Leave the system powered on for 24 hours with the fridge operating normally.

| Check | Pass condition |
| --- | --- |
| Thinger.io data continuity | No gaps longer than 30 seconds in the time series chart |
| Memory stability | No resets or crashes visible in Serial Monitor logs |
| Temperature accuracy | Readings stay within 1C of a reference thermometer |
| Door alert | Alert fires every time door is held open more than 30 seconds |
| OCR captures | At least 5 captures taken and results published to dashboard |
| Weight stability | Weight reading does not drift more than 0.05 kg over 24 hours |